AUDIO MUSINGS

Vocabulary Atlas — Session 1: What Is Sound?

An expandable reference connecting acoustic physics, auditory perception, recording practice, and digital-audio technology.

| Edition | Session 1 foundation |
| --- | --- |
| Entry format | Definition • Digital relationship • Best use • Technology |

# How to use this glossary

Each entry keeps the physical meaning separate from its digital representation. “Digital relationship” explains how software measures, stores, models, or manipulates the phenomenon. “Best use” connects the concept to practice, while “Technology” points toward tools worth learning or building.

This is a living Audio Musings reference. Future sessions can add new categories, cross-references, formulas, listening exercises, and project notes without changing the entry structure.

# Category map

1. Foundations of Sound (8 terms)

2. Wave Properties and Measurement (11 terms)

3. Acoustic Interaction and Propagation (13 terms)

4. Resonance, Rooms, and Spatial Behavior (10 terms)

5. Spectrum, Tone, and Timbre (12 terms)

6. Time Behavior, Dynamics, and Perception (13 terms)

7. Digital Audio and Signal Technology (18 terms)

8. Recording, Monitoring, and Practical Control (8 terms)

# 1. Foundations of Sound

## Sound

Definition  Mechanical energy traveling through matter as a pressure or structural disturbance.

Digital relationship  Captured as an electrical signal and represented digitally as ordered amplitude samples.

Best use  Recording, synthesis, playback, measurement, and analysis.

Technology  Microphones, audio interfaces, DAWs, speakers, oscilloscopes.

## Vibration

Definition  Repeated movement around an equilibrium position that can transfer energy into a medium.

Digital relationship  Represented by changing sample values; modeled with oscillators and modulation signals.

Best use  Generating tones, analyzing machinery, synthesis, and speaker design.

Technology  Oscillators, accelerometers, contact microphones, waveform generators.

## Medium

Definition  Matter through which a mechanical wave travels, such as air, water, wood, or steel.

Digital relationship  Usually simulated through acoustic models rather than stored directly in an audio file.

Best use  Predicting propagation, isolation, underwater audio, and structural noise.

Technology  Acoustic simulation, finite-element modeling, hydrophones, vibration sensors.

## Pressure Wave

Definition  Alternating changes in pressure that propagate through a medium.

Digital relationship  A microphone converts pressure variation into voltage, then an ADC converts voltage into samples.

Best use  Recording, SPL measurement, loudspeaker testing, and acoustic sensing.

Technology  Measurement microphones, ADCs, SPL meters, pressure transducers.

## Compression (Acoustic)

Definition  A region where particles are temporarily crowded and pressure rises above equilibrium.

Digital relationship  Appears as one polarity of a recorded waveform; not the same as dynamic-range compression.

Best use  Understanding speaker motion, microphone polarity, and wave propagation.

Technology  Microphones, oscilloscopes, polarity tests, acoustic simulators.

## Rarefaction

Definition  A region where particles spread apart and pressure falls below equilibrium.

Digital relationship  Appears as the opposite waveform polarity from compression.

Best use  Understanding wave cycles, phase, polarity, and transducer behavior.

Technology  Waveform displays, oscilloscopes, microphones, speaker test systems.

## Source

Definition  The vibrating object or system that initiates a sound disturbance.

Digital relationship  May be recorded, sampled, synthesized, or represented as a virtual sound source.

Best use  Source identification, recording placement, mixing, and sound design.

Technology  Instruments, speakers, samplers, synthesizers, source-separation software.

## Receiver

Definition  A system that detects acoustic energy, such as an ear, microphone, or sensor.

Digital relationship  Produces data that can be sampled, analyzed, transmitted, or stored.

Best use  Recording, monitoring, measurement, localization, and machine listening.

Technology  Microphones, hydrophones, contact mics, binaural heads, sensor arrays.

# 2. Wave Properties and Measurement

## Frequency

Definition  The number of wave cycles per second, measured in hertz (Hz).

Digital relationship  Mapped to spectral bins by Fourier analysis and controlled by oscillators, filters, and pitch tools.

Best use  Pitch control, EQ, tuning, crossover design, and fault detection.

Technology  Spectrum analyzers, FFTs, tuners, EQs, synthesizers.

## Hertz (Hz)

Definition  The unit meaning cycles per second.

Digital relationship  Used to label sample rates, tone frequencies, filters, and spectral measurements.

Best use  Specifying pitch, bandwidth, cutoff points, and sampling systems.

Technology  DAWs, analyzers, oscillators, filters, audio interfaces.

## Period

Definition  The time required for one complete cycle; period equals 1 divided by frequency.

Digital relationship  Measured in seconds, milliseconds, or samples and used for delay and oscillator timing.

Best use  Phase alignment, synthesis, looping, and calculating wavelength.

Technology  Sample editors, oscilloscopes, delay calculators, DSP code.

## Amplitude

Definition  The magnitude of a vibration or pressure variation.

Digital relationship  Stored as sample magnitude and adjusted by gain, faders, envelopes, and dynamics processors.

Best use  Controlling level, shaping transients, modulation, and preventing clipping.

Technology  Faders, gain stages, compressors, limiters, waveform editors.

## Wavelength

Definition  The physical length of one cycle; speed of sound divided by frequency.

Digital relationship  Used in acoustic calculators and spatial models rather than encoded as an audio-file field.

Best use  Room treatment, speaker placement, microphone arrays, and bass management.

Technology  Acoustic calculators, simulation tools, measurement systems.

## Phase

Definition  A wave's position within its cycle relative to another reference.

Digital relationship  Represented by timing and angular relationships; altered by delays, filters, and phase rotation.

Best use  Aligning microphones and subs, layering samples, widening, and cancellation control.

Technology  Phase meters, correlation meters, delay tools, all-pass filters.

## Polarity

Definition  The positive-versus-negative orientation of a signal or transducer connection.

Digital relationship  A polarity invert multiplies every sample by -1; it does not create a time shift.

Best use  Correcting reversed wiring and testing multi-microphone or speaker systems.

Technology  Polarity switches, DAW utility plug-ins, cable testers.

## Waveform

Definition  A representation of how a signal's amplitude changes over time.

Digital relationship  Displayed from stored samples and edited directly in sample editors and DAWs.

Best use  Editing, transient inspection, looping, clipping detection, and sound design.

Technology  DAWs, sample editors, oscilloscopes, waveform viewers.

## Speed of Sound

Definition  The propagation speed of a sound wave through a particular medium.

Digital relationship  Used by delay, distance, beamforming, and acoustic-model calculations.

Best use  Microphone alignment, echo location, room measurement, and spatial audio.

Technology  Measurement software, DSP algorithms, acoustic simulators.

## Sound Pressure Level (SPL)

Definition  A logarithmic measure of acoustic pressure relative to a reference, expressed in dB SPL.

Digital relationship  Measured by calibrated sensors; software can log, average, and visualize the readings.

Best use  Monitoring exposure, calibrating studios, measuring speakers, and noise surveys.

Technology  SPL meters, calibrated microphones, acoustic-analysis software.

## Decibel (dB)

Definition  A logarithmic unit expressing a ratio, with meaning determined by its reference.

Digital relationship  Appears as dBFS, LUFS-related readings, dBu, dBV, gain changes, and meter scales.

Best use  Gain staging, loudness control, calibration, dynamics, and system specification.

Technology  Meters, DAWs, consoles, analyzers, audio interfaces.

# 3. Acoustic Interaction and Propagation

## Reflection

Definition  Sound energy that returns from a boundary instead of being fully absorbed or transmitted.

Digital relationship  Modeled by delay taps, convolution, early-reflection engines, and ray tracing.

Best use  Creating spatial realism, measuring rooms, positioning treatment, and designing reverbs.

Technology  Convolution reverbs, room simulators, impulse-response tools, acoustic panels.

## Absorption

Definition  Conversion of acoustic energy into small amounts of heat within a material.

Digital relationship  Represented by frequency-dependent absorption coefficients in acoustic models.

Best use  Reducing reflections, flutter echo, and room decay without necessarily soundproofing.

Technology  Porous panels, mineral wool, acoustic modeling, reverberation measurement.

## Transmission

Definition  Sound energy passing through a material or structure into another space.

Digital relationship  Measured and modeled with transmission loss and frequency-dependent isolation data.

Best use  Soundproofing, enclosure design, privacy, and controlling equipment noise.

Technology  STC/TL testing, acoustic simulation, isolation construction, measurement mics.

## Diffraction

Definition  Bending and spreading of waves around obstacles and through openings.

Digital relationship  Simulated in spatial-audio engines and boundary/finite-element models.

Best use  Speaker cabinet design, array behavior, architectural acoustics, and mic placement.

Technology  Acoustic simulators, wave-field models, loudspeaker design software.

## Refraction

Definition  Change in wave direction caused by a change in propagation speed.

Digital relationship  Modeled using temperature, density, depth, or medium gradients.

Best use  Outdoor sound prediction, underwater acoustics, and atmospheric measurement.

Technology  Environmental acoustic models, sonar systems, weather-linked analysis.

## Scattering

Definition  Redistribution of sound in many directions by irregular surfaces or objects.

Digital relationship  Approximated with diffusion/scattering coefficients and spatial models.

Best use  Reducing harsh reflections and improving spatial uniformity.

Technology  Diffusers, room-modeling software, impulse-response measurement.

## Attenuation

Definition  Reduction in sound energy as it travels or passes through a system.

Digital relationship  Implemented as gain reduction and modeled as distance- or frequency-dependent loss.

Best use  Level control, distance simulation, cable/system design, and hearing protection.

Technology  Gain controls, filters, acoustic models, attenuators.

## Inverse-Square Law

Definition  In free-field conditions, intensity falls with the square of distance from a point source.

Digital relationship  Used by virtual acoustics and game-audio distance curves, often with practical modifications.

Best use  Predicting level changes with distance and placing microphones or speakers.

Technology  Game-audio engines, acoustic calculators, SPL measurement tools.

## Airborne Sound

Definition  Sound transmitted primarily through the air.

Digital relationship  Captured by conventional microphones and analyzed as an acoustic waveform.

Best use  Voice and music recording, environmental monitoring, and room treatment.

Technology  Condenser/dynamic microphones, SPL meters, spectral analyzers.

## Structure-Borne Sound

Definition  Mechanical vibration traveling through solids before radiating as audible sound.

Digital relationship  Captured with vibration sensors or contact microphones and analyzed as time-series data.

Best use  Diagnosing hum, isolating turntables, machinery monitoring, and building acoustics.

Technology  Accelerometers, contact mics, isolation mounts, vibration-analysis software.

## Flanking Path

Definition  An indirect route by which sound bypasses the intended barrier.

Digital relationship  Investigated through multi-point measurement and building-acoustic models.

Best use  Finding why a supposedly isolated room still leaks sound.

Technology  Measurement microphones, vibration sensors, acoustic cameras, modeling tools.

## Mechanical Decoupling

Definition  Reducing vibration transfer by separating connected structures or components.

Digital relationship  Evaluated through before/after spectral and vibration measurements.

Best use  Speaker isolation, studio construction, turntable protection, and quiet hardware.

Technology  Isolation pads, resilient channels, shock mounts, accelerometers.

## Damping

Definition  Dissipation of vibrational energy that reduces resonance and ringing.

Digital relationship  Modeled with decay constants and applied through resonator or filter parameters.

Best use  Controlling panels, speaker cabinets, drums, hardware vibration, and feedback.

Technology  Constrained-layer damping, dampers, transient analyzers, modal software.

# 4. Resonance, Rooms, and Spatial Behavior

## Resonance

Definition  A strong response when a system is excited near one of its natural frequencies.

Digital relationship  Appears as peaks and prolonged decay in spectra, waterfall plots, and impulse responses.

Best use  Tuning instruments, diagnosing rooms, designing enclosures, and creative filtering.

Technology  Spectrum analyzers, resonators, parametric EQ, room-measurement software.

## Natural Frequency

Definition  A frequency at which a physical system tends to vibrate freely.

Digital relationship  Estimated from recorded responses or modeled numerically.

Best use  Avoiding mechanical failure, tuning instruments, and controlling cabinet or room vibration.

Technology  Modal analysis, FFT software, accelerometers, measurement mics.

## Standing Wave

Definition  A stable interference pattern created by waves traveling in opposing directions.

Digital relationship  Seen as location-dependent peaks/nulls and modeled from geometry and boundary conditions.

Best use  Understanding bass inconsistencies and positioning speakers, subs, and listeners.

Technology  Room calculators, measurement microphones, bass-management software.

## Room Mode

Definition  A standing-wave resonance determined by a room's dimensions and boundaries.

Digital relationship  Calculated by software and confirmed with frequency sweeps and decay plots.

Best use  Bass treatment, subwoofer placement, room correction, and mix-position selection.

Technology  REW-style measurement software, calibrated mics, DSP room correction.

## Node and Antinode

Definition  Locations of minimum and maximum motion or pressure in a standing wave.

Digital relationship  Mapped through repeated spatial measurements or acoustic simulation.

Best use  Locating bass nulls/peaks and placing treatment, microphones, and subwoofers.

Technology  SPL mapping, room simulators, microphone arrays.

## Reverberation

Definition  The dense persistence of reflected sound after the source stops.

Digital relationship  Created by algorithmic reverb or convolution and measured using decay time.

Best use  Establishing space, depth, cohesion, realism, or dramatic scale.

Technology  Reverb plug-ins, convolution engines, impulse-response libraries.

## Echo

Definition  A reflection heard as a distinct repetition rather than fused into reverberation.

Digital relationship  Created with delay lines and feedback networks.

Best use  Rhythmic effects, spatial cues, dub production, and distance simulation.

Technology  Delay plug-ins, tape echoes, digital delay units.

## Reverberation Time (RT60)

Definition  The time required for sound level in a room to decay by 60 dB.

Digital relationship  Calculated from impulse responses or decay measurements.

Best use  Evaluating rooms for speech, music, recording, and treatment decisions.

Technology  Measurement software, swept-sine tests, calibrated microphones.

## Comb Filtering

Definition  A series of regularly spaced spectral peaks and notches caused by delayed signal combination.

Digital relationship  Produced when a signal is mixed with a short delayed copy.

Best use  Diagnosing reflections and mic phase; creatively used in flanging and filtering.

Technology  Spectrum analyzers, delay lines, flangers, phase-alignment tools.

## Free Field

Definition  An ideal region where direct sound dominates and boundaries contribute negligible reflections.

Digital relationship  Approximated in simulations and anechoic measurements.

Best use  Loudspeaker testing, distance calculations, and reference measurements.

Technology  Anechoic chambers, outdoor measurement, acoustic simulators.

# 5. Spectrum, Tone, and Timbre

## Pure Tone

Definition  A sinusoidal vibration containing one frequency.

Digital relationship  Generated mathematically by a sine oscillator.

Best use  Calibration, hearing tests, synthesis, and system troubleshooting.

Technology  Signal generators, synthesizers, test-tone plug-ins.

## Fundamental Frequency

Definition  The lowest periodic component strongly associated with a sound's perceived pitch.

Digital relationship  Detected or estimated by pitch trackers and spectral algorithms.

Best use  Tuning, transcription, pitch correction, synthesis, and sample organization.

Technology  Tuners, pitch detectors, autocorrelation, FFT analysis.

## Harmonic

Definition  A frequency component at an integer multiple of a fundamental.

Digital relationship  Measured in spectra and generated through oscillation, saturation, or nonlinear processing.

Best use  Shaping timbre, synthesis, distortion, instrument analysis, and bass enhancement.

Technology  Spectrum analyzers, saturators, additive synthesizers.

## Overtone

Definition  Any frequency component above the fundamental; it may be harmonic or inharmonic.

Digital relationship  Displayed and measured through spectral analysis.

Best use  Describing instrument character, resonance, and complex sound design.

Technology  Spectrograms, FFT analyzers, resynthesis tools.

## Inharmonicity

Definition  Departure of partial frequencies from exact whole-number multiples of a fundamental.

Digital relationship  Measured by spectral peak analysis and modeled in physical synthesis.

Best use  Designing bells and metallic sounds, piano tuning, and instrument identification.

Technology  Modal synthesis, spectral analyzers, physical-modeling synths.

## Timbre

Definition  The qualities that distinguish sounds sharing similar pitch and loudness.

Digital relationship  Represented through spectral shape, envelope, modulation, noise, and temporal features.

Best use  Instrument choice, synthesis, mixing, classification, and sample search.

Technology  Synthesizers, samplers, spectral tools, audio embeddings.

## Spectrum

Definition  The distribution of a sound's energy across frequency.

Digital relationship  Computed with Fourier transforms and displayed as magnitude versus frequency.

Best use  EQ decisions, noise diagnosis, timbre analysis, and mastering.

Technology  FFT spectrum analyzers, spectrograms, EQ displays.

## Fourier Analysis

Definition  A mathematical method that decomposes complex signals into sinusoidal components.

Digital relationship  Foundation of FFT analysis, spectral processing, convolution, and many codecs.

Best use  Measurement, filtering, restoration, pitch analysis, and sound visualization.

Technology  FFT libraries, DAWs, analyzers, spectral editors.

## Frequency Domain

Definition  A representation emphasizing frequency content rather than change over time.

Digital relationship  Displays magnitude and sometimes phase across frequency bins.

Best use  EQ, noise removal, harmonic analysis, and acoustic measurement.

Technology  Spectrum analyzers, spectrograms, spectral editors.

## Time Domain

Definition  A representation of signal amplitude as it changes through time.

Digital relationship  The ordinary sample-by-sample waveform view used by DAWs.

Best use  Editing, transient inspection, synchronization, dynamics, and clipping detection.

Technology  Waveform editors, oscilloscopes, DAWs.

## Noise

Definition  Aperiodic or complex energy spread across frequencies; not inherently unwanted.

Digital relationship  Generated, recorded, filtered, classified, reduced, or used as a measurement signal.

Best use  Percussion, ambience, dither, synthesis, masking, and system testing.

Technology  Noise generators, denoisers, spectrograms, measurement systems.

## Beating

Definition  Audible amplitude fluctuation produced by interference between nearby frequencies.

Digital relationship  Generated by combining close oscillators or detuning voices.

Best use  Tuning instruments, chorus effects, tension, and motion in synthesis.

Technology  Tuners, oscillators, chorus plug-ins, spectrum analyzers.

# 6. Time Behavior, Dynamics, and Perception

## Envelope

Definition  The overall change in a sound's amplitude over time.

Digital relationship  Extracted by envelope followers and shaped with ADSR controls or automation.

Best use  Synthesis, compression, transient design, side-chaining, and sample matching.

Technology  Envelope followers, ADSR generators, compressors, DAW automation.

## Attack

Definition  The initial rise of a sound from silence or baseline level.

Digital relationship  Controlled by envelopes, transient shapers, compressor attack, and fades.

Best use  Creating punch, softness, articulation, and rhythmic definition.

Technology  ADSR controls, transient shapers, compressors, sample editors.

## Decay

Definition  The reduction in level after the initial peak or excitation.

Digital relationship  Measured and shaped with envelopes, gates, damping models, and reverb controls.

Best use  Controlling drum length, room tails, resonance, and perceived size.

Technology  ADSR generators, gates, reverbs, transient processors.

## Sustain

Definition  The maintained portion of a sound while energy or control continues.

Digital relationship  Specified as an envelope level and extended through looping or synthesis.

Best use  Pads, notes, sampling, instrument modeling, and vocal/instrument shaping.

Technology  ADSR envelopes, samplers, loop tools, synthesizers.

## Release

Definition  The final decay after excitation or a control signal stops.

Digital relationship  Controlled by envelope release, gate release, fades, and reverb tails.

Best use  Natural note endings, smooth gating, ambience, and preventing clicks.

Technology  ADSR envelopes, gates, samplers, automation.

## Transient

Definition  A brief, high-energy change—often the identifying attack of a sound.

Digital relationship  Detected using amplitude/spectral change algorithms and edited at sample precision.

Best use  Drum replacement, slicing, timing, compression, and punch control.

Technology  Transient detectors, slicers, transient shapers, DAWs.

## Pitch

Definition  The perceptual ordering of sound from low to high.

Digital relationship  Estimated from periodicity and manipulated by pitch shifting, tuning, or resampling.

Best use  Melody, tuning, harmonic mixing, vocal correction, and sample design.

Technology  Pitch trackers, Auto-Tune-style tools, samplers, phase vocoders.

## Loudness

Definition  The perceived strength of sound, influenced by level, frequency, duration, and context.

Digital relationship  Estimated with loudness algorithms such as LUFS measurements.

Best use  Broadcast delivery, mastering, platform consistency, and safe monitoring.

Technology  Loudness meters, limiters, normalization systems.

## Masking

Definition  Reduced audibility of one sound because another occupies nearby time or frequency regions.

Digital relationship  Predicted by perceptual codecs and managed with EQ, arrangement, or source separation.

Best use  Mix clarity, data compression, noise design, and psychoacoustic experiments.

Technology  EQs, codecs, source-separation tools, perceptual analyzers.

## Localization

Definition  The auditory estimation of a sound source's direction and distance.

Digital relationship  Recreated using interaural timing/level differences, HRTFs, and spatial metadata.

Best use  Headphone spatial audio, VR, games, binaural recording, and speaker arrays.

Technology  Binaural processors, HRTFs, ambisonics, microphone arrays.

## Psychoacoustics

Definition  The study of relationships between physical sound and auditory perception.

Digital relationship  Guides codecs, loudness models, spatial audio, masking tools, and interface design.

Best use  Efficient coding, better mixes, hearing research, and perceptually informed products.

Technology  Perceptual codecs, LUFS meters, hearing-test software, spatial engines.

## Consonance

Definition  A perceived quality of stability or compatibility among tones, shaped by acoustics and culture.

Digital relationship  Analyzed through interval ratios, roughness models, and harmonic relationships.

Best use  Harmony, tuning systems, arrangement, generative music, and music-information retrieval.

Technology  MIDI/DAWs, tuning software, harmonic analyzers, composition systems.

## Dissonance

Definition  A perceived quality of tension, roughness, or instability among sounds.

Digital relationship  Modeled through beating, spectral roughness, tuning, and contextual musical rules.

Best use  Creating tension, color, motion, experimental texture, and emotional contrast.

Technology  Synthesizers, microtonal tools, spectral analyzers, composition software.

# 7. Digital Audio and Signal Technology

## Audio Signal

Definition  A changing electrical or numerical representation corresponding to sound.

Digital relationship  In digital systems it is a sequence of sample values organized into channels.

Best use  Recording, routing, processing, transmission, and playback.

Technology  Audio interfaces, DAWs, mixers, DSP systems.

## Transducer

Definition  A device that converts energy from one form to another.

Digital relationship  Connects acoustic and electrical/digital domains through microphones and speakers.

Best use  Capturing and reproducing sound or vibration.

Technology  Microphones, speakers, pickups, headphones, accelerometers.

## Analog-to-Digital Converter (ADC)

Definition  A circuit that converts an analog voltage into numerical samples.

Digital relationship  Defines sample rate, bit depth, noise, distortion, and input-level constraints.

Best use  Recording microphones, instruments, vinyl, and measurement signals.

Technology  Audio interfaces, field recorders, measurement hardware.

## Digital-to-Analog Converter (DAC)

Definition  A circuit that converts numerical samples into a continuous electrical signal.

Digital relationship  Feeds amplifiers, headphones, monitors, or other analog equipment.

Best use  Monitoring, playback, mastering, and equipment testing.

Technology  Audio interfaces, headphone DACs, controllers, digital mixers.

## Sample

Definition  One numerical measurement of signal amplitude at a particular moment.

Digital relationship  The fundamental time-domain unit of pulse-code-modulated digital audio.

Best use  Editing, DSP, synthesis, storage, and analysis.

Technology  DAWs, audio files, DSP libraries, samplers.

## Sample Rate

Definition  The number of samples captured or reproduced per second.

Digital relationship  Determines the highest representable frequency under sampling theory and affects processing bandwidth.

Best use  Choosing recording formats, designing DSP, resampling, and video synchronization.

Technology  Audio interfaces, DAWs, sample-rate converters.

## Bit Depth

Definition  The number of bits used to represent each sample's amplitude.

Digital relationship  Influences quantization resolution and available digital dynamic range.

Best use  Recording with headroom, selecting file formats, and controlling noise in processing.

Technology  ADCs, DAWs, WAV files, dither processors.

## Dynamic Range

Definition  The span between a system's noise floor and its maximum usable level.

Digital relationship  Limited by converters, bit depth, analog electronics, processing, and delivery formats.

Best use  Gain staging, recording, mastering, and preserving expressive level differences.

Technology  Meters, interfaces, compressors, limiters, analyzers.

## dBFS

Definition  A digital level scale referenced to the maximum representable sample value.

Digital relationship  0 dBFS is the nominal digital ceiling; lower levels use negative values.

Best use  Preventing clipping, gain staging, metering, and delivery checks.

Technology  Peak meters, DAWs, limiters, audio editors.

## LUFS

Definition  A standardized estimate of perceived program loudness relative to digital full scale.

Digital relationship  Calculated using frequency weighting and time integration across audio.

Best use  Mastering for streaming, broadcast compliance, and loudness matching.

Technology  LUFS meters, mastering suites, platform-analysis tools.

## Clipping

Definition  Distortion caused when a system cannot represent or pass a signal's peaks.

Digital relationship  Digital hard clipping truncates sample values at the available limit.

Best use  Troubleshooting harsh distortion; sometimes used deliberately as an effect.

Technology  Peak meters, oscilloscopes, clippers, limiters.

## Quantization

Definition  Mapping a continuous amplitude to one of a finite number of numerical values.

Digital relationship  Occurs during analog-to-digital conversion and bit-depth reduction.

Best use  Understanding converter resolution, distortion, and low-level signal behavior.

Technology  ADCs, bit-depth processors, DSP systems.

## Dither

Definition  Low-level noise added before reducing bit depth to decorrelate quantization error.

Digital relationship  Turns deterministic distortion into less objectionable broadband noise.

Best use  Final bit-depth conversion, especially for master delivery.

Technology  Dither plug-ins, mastering software, export engines.

## Digital Signal Processing (DSP)

Definition  Mathematical manipulation of sampled signals.

Digital relationship  Implements filters, dynamics, effects, analysis, coding, synthesis, and restoration.

Best use  Nearly every modern audio workflow, from EQ to machine listening.

Technology  CPUs, DSP chips, plug-ins, Python/C++ audio libraries.

## Filter

Definition  A system that changes selected frequency components of a signal.

Digital relationship  Implemented with digital algorithms such as FIR or IIR structures.

Best use  EQ, crossover design, noise removal, synthesis, and anti-aliasing.

Technology  EQ plug-ins, synthesizers, DSP libraries, crossovers.

## Impulse Response

Definition  A system's output after a very brief standardized excitation.

Digital relationship  Stored as audio data and used by convolution to reproduce a system's behavior.

Best use  Capturing rooms, reverbs, speakers, cabinets, and filters.

Technology  Convolution reverbs, measurement software, IR libraries.

## Convolution

Definition  A mathematical operation combining a signal with an impulse response.

Digital relationship  Applied digitally to reproduce measured reverberation or system coloration.

Best use  Realistic room reverb, cabinet simulation, restoration, and filtering.

Technology  Convolution plug-ins, DSP libraries, IR processors.

## Latency

Definition  Delay between an input event and the corresponding output.

Digital relationship  Created by buffering, conversion, processing, operating systems, and transmission.

Best use  Live monitoring, DJ performance, recording, networked audio, and interactive instruments.

Technology  Audio drivers, low-latency interfaces, buffer controls, network-audio protocols.

# 8. Recording, Monitoring, and Practical Control

## Microphone

Definition  A transducer that converts sound pressure or vibration into an electrical signal.

Digital relationship  Its output is amplified and digitized for storage and processing.

Best use  Vocals, instruments, ambience, measurement, and sound-event detection.

Technology  Dynamic, condenser, ribbon, contact, and measurement microphones.

## Polar Pattern

Definition  A microphone's direction-dependent sensitivity.

Digital relationship  Visualized as a polar plot and sometimes modeled in virtual microphone systems.

Best use  Rejecting noise, controlling room pickup, stereo recording, and feedback management.

Technology  Cardioid/omni/figure-8 microphones, modeling mics, array DSP.

## Preamplifier

Definition  A circuit that raises a low-level source signal to a workable level.

Digital relationship  Sets the analog level before conversion and strongly affects noise and headroom.

Best use  Recording microphones and low-output instruments cleanly.

Technology  Audio interfaces, mixer preamps, standalone preamps.

## Gain Staging

Definition  Managing signal levels through every stage of an audio chain.

Digital relationship  Keeps digital and analog stages within useful headroom and noise limits.

Best use  Avoiding clipping/noise and maintaining predictable plug-in behavior.

Technology  Meters, preamps, faders, trims, reference-level calibration.

## Feedback

Definition  A loop in which output re-enters a system's input and is amplified again.

Digital relationship  Modeled with feedback paths in delays, filters, and control systems.

Best use  Avoiding live howling or creatively generating echoes and sustained tones.

Technology  Mixers, feedback suppressors, delay plug-ins, routing matrices.

## Acoustic Treatment

Definition  Modification of a room's internal reflections, decay, and frequency behavior.

Digital relationship  Designed and verified with measurements, models, and before/after impulse responses.

Best use  Improving recording clarity and monitoring accuracy.

Technology  Broadband absorbers, bass traps, diffusers, room-measurement software.

## Sound Isolation

Definition  Reduction of sound transmission between spaces or structures.

Digital relationship  Evaluated with level differences, transmission-loss data, and vibration measurements.

Best use  Privacy, neighbor control, quiet recording, and machinery containment.

Technology  Mass barriers, decoupled construction, seals, measurement systems.

## Acoustic Treatment vs. Soundproofing

Definition  Treatment changes sound within a room; soundproofing reduces transfer between rooms.

Digital relationship  Digital measurement can diagnose both, but plug-ins cannot physically isolate a room.

Best use  Choosing the correct solution before buying foam and discovering betrayal.

Technology  Room measurement, absorbers/diffusers, mass, seals, decoupling.

# Session 1 relationship map

A practical way to remember the vocabulary is to follow sound through a complete system:

A source vibrates and creates compressions and rarefactions in a medium.

The pressure wave travels with measurable frequency, amplitude, wavelength, period, and phase.

Boundaries divide its energy among reflection, absorption, transmission, diffraction, and scattering.

Rooms add resonance, standing waves, modes, echoes, reverberation, and comb filtering.

A microphone and preamplifier convert and condition the event; an ADC creates digital samples.

DSP analyzes or changes the signal in the time and frequency domains.

A DAC and speaker return the samples to mechanical motion and acoustic pressure.

The auditory system interprets pitch, loudness, timbre, location, consonance, and dissonance.

# Future expansion tags

Suggested tags for later entries: #acoustics, #psychoacoustics, #recording, #mixing, #mastering, #sampling, #synthesis, #DJing, #vinyl, #soundproofing, #room-treatment, #DSP, #spatial-audio, #machine-listening, #audio-ML, #hardware, #measurement, and #hearing-safety.